## Maseeh College of Engineering and Computer Science

Department of Civil and Environmental Engineering

Post Office Box 751	503-725-4282 tel

Portland, Oregon 97207-0751	503-725-5950 fax

CE443(543) Introduction to Geotechnical Earthquake Engineering

To:         CE443(543) Students							Date: February 12, 2026

From:     Arash Khosravifar

Subject: Homework Assignment #5 (Due Feb 20th at 11:59 p.m.)

Problem 1:

Consider the isotropically consolidated drained triaxial compression tests for specimens with an initial relative density of 100% by Lee and Seed (1967) in Figure 9 of the Monograph (Idriss and Boulanger, 2008; copied below).

- Plot the peak effective friction angle versus effective confining stress (σ3c'). Above what effective confining stress does the peak effective friction angle become approximately constant for these specimens (i.e., equal to the constant volume friction angle)?

- On one figure, plot the Mohr circles at peak deviator stress for effective confining stresses of about 100, 290, and 1000 kPa. Draw a Mohr Coulomb failure surface to fit these test results (not as a straight line, but as an appropriately curved surface).

- Compute the final void ratio for both specimens (DR=100% and 38%) and all confining effectives stresses (σ3c') on Figure 9, assuming that the reported volumetric strains are uniform throughout the specimens. Assume emin = 0.61 and emax = 1.03 for this sand. On one figure, plot the final void ratio versus initial confining stress. Do the computed void ratios converge to the same "critical state" void ratio at large strains? If not, explain why.

- Estimate the peak deviatoric stress that can be carried by a specimen with a relative density of 100% tested in ICD triaxial compression with an effective confining stress of 500 kPa (you can interpolate the data in the top left plot on Figure 9).

- If a specimen at DR=100% with an effective consolidation stress of 300 kPa was tested in undrained compression, would the pore water pressure be expected to increase or decrease? Would this sample be initially dense or loose of critical state? Explain your reasoning.

Hints:

Part (a): The peak effective friction angle for this sand should plateau at about 33 degrees at large effective stresses (larger than ~40 atm)

Part (c): For example, at confining stress of 116 atm, the loose sample (Dr=38%) has an initial void ratio of eo=0.87, final volumetric strain of εvol=-15.7%, and final void ratio of ef=0.576.

Part (d): qpeak = Δσd,peak = 1750 kPa

Part (e): dense of critical expected to dilate.

Problem 2:

Consider the undrained triaxial compression test result for specimen B of Toyoura sand in Figure 12 (Ishihara 1993) of the Monograph:

- What is the constant volume (or critical state) effective friction angle, φcv'?

- On a single figure, plot the Mohr circles for the specimen at the following times: (i) the end of consolidation, (ii) at phase transformation (i.e., the transition from incrementally contractive to incrementally dilative behavior), and (iii) at critical state.

- Determine the excess pore pressure (Δu) at point Q.

Hints:

Part (a): 30.9 degrees

Part (c): 71 kPa

Problem 3:

Consider the undrained triaxial compression test data for Toyoura sand summarized in Figure 12 of the Monograph.

- For a specimen that was isotropically consolidated at an effective confining stress of 200 kPa and an initial void ratio of 0.920, and then tested in undrained compression, estimate the critical state values for the following quantities: σ3', σ1', Δu, ru, p' and q.

- For a specimen that was isotropically consolidated at an effective confining stress of 200 kPa and an initial void ratio of 0.880, and then tested in undrained compression, estimate the critical state values for the following quantities: σ3', σ1', Δu, ru, p', and q.

- If the specimens in (a) and (b) had been loaded in drained triaxial compression, would they have been contractive, dilative, or not changed volume at all? Would they be stronger in drained or undrained loading?

Hints:

Part (a): Δu = 164 kPa

Part (b): Δu = -29 kPa

Part (c): Sample B is dense of critical, dilative, negative PWP in undrained, stronger in undrained compared to drained.